# PUMPKINS input list

All inputs are connected to the IEM patch bay; parallel XLR snake is provided

- 2x kick, 2x snare, 5x toms, 2x H-H, lot of cymbals

- 2x stereo guitar profiler (XLR with GND lift)

- 1x stereo mixed keyboards (XLR), 4x DI box in case of need

- Guitars are main instruments, keyboards must not be louder (except solos)

- Own Beyerdynamic drum microphones, please have 2x B91A as a backup for kicks and something decent for the rest of the kit (high SPL rated mics)

- Stage Monitors (Wedges): We provide 3 pre-mixed mono feeds from our system (labeled as SEND 1–3, XLR Male). Please connect these lines directly to your stage monitors. No monitor mixing is required from the FOH console.

- FOH FX Requirements:

  - FX 1 (Drum Hall): For SD and FX SD. For outdoor venues, please apply to all drum shells.

  - FX 2 (Gated Reverb): For SD.

  - FX 3 (Vocal Hall): General hall for all vocals in the mix.

  - FX 4 (Vocal Delay): Dedicated for Lead Vocal (Voc 2), tap-tempo required.

| Snake. | Local IN | Name | Mic/line | Notes |
| --- | --- | --- | --- | --- |
| 1 |  | Kick 1 in | Own D71 |  |
| 2 |  | Kick 2 in | Own D71 |  |
| 3 |  | SD Top | Own i51 |  |
| 4 |  | SD Bot | Own d35d |  |
| 5 |  | TT 1 | Own d58c |  |
| 6 |  | TT 2 | Own d58c |  |
| 7 |  | TT 3 | Own d58c |  |
| 8 |  | Floor T 1L | Own d57c |  |
| 9 |  | Floor T 2L | Own d57c |  |
| 10 |  | FX SD 12” | Own d35d |  |
| 11 |  | OH L | Own i53c |  |
| 12 |  | OH R | Own i53c |  |
| 13 |  | Bass | Line (DI) |  |
| 14 |  | SPARE |  | reserved |
| 15 |  | Keys 1 L | Line (DI) |  |
| 16 |  | Keys 1 R | Line (DI) |  |
| 17 |  | GTR 1 L | Line |  |
| 18 |  | GTR 1 R | Line |  |
| 19 |  | GTR 2 L | Line |  |
| 20 |  | GTR 2 R | Line |  |
| 21 |  | Voc 1 (left GTR) | Own e945 |  |
| 22 |  | Voc 2 (lead) | Own e945 | wireless |
| 23 |  | Voc 3 (keys) | Own e945 |  |
| 24 |  | Voc 4 (drums) | Own beta56a |  |
| AUX1 |  | FOH Talkback | AUX1 |  |
| SEND 1 |  | Mix Wedge L |  | Pre-mixed mono wedge mix -> Connect directly to stage wedge |
| SEND 2 |  | Mix Wedge C |  |  |
| SEND 3 |  | Mix Wedge R |  |  |